OEE Platform

Parameter Reference

Live Machine Data — IIoT Gateway JS Script

These parameters are collected by the JS Script running on the IIoT Gateway. The script polls the FANUC controller every 5 seconds and sends the complete JSON payload to the VM where it is stored in the cnc_data TimescaleDB hypertable.

Push frequency: Every 5 seconds  |  Destination: VM → Thingsboard via mqtt

| Parameter | Meaning | Source / How Derived |
| --- | --- | --- |
| timestamp | ISO 8601 timestamp of the reading | VM system clock at time of insertion |
| machine_state | Current controller state: RUNNING / STOPPED / ALARM / IDLE | FANUC PMC signal — status word register |
| auto_mode | Operating mode: MEMORY / MDI / JOG / HANDLE | FANUC CNC mode register (cncmode) |
| run_status | Execution status: MSTR (running) / STOP / RESET | FANUC run status word (run_stat) |
| is_running | Boolean — true when machine is actively cutting (state = RUNNING) | Derived from machine_state in JS Script |
| emergency | Emergency stop engaged (true/false) | FANUC E-stop relay signal via PMC |
| alarm_active | Any alarm currently active on controller (true/false) | FANUC alarm register — true if alarm_no > 0 |
| alarm_no | Active alarm code number (e.g. 101 = servo error, 0 = no alarm) | FANUC alarm number register (rdalmsg) |
| motion_status | Axis motion state: CUTTING / RAPID / DWELL / STOP | FANUC motion state via modal G-code group |
| program_no | O-number of CNC program currently loaded and running | FANUC active program register (execprog) |
| feed_rate | Actual current feed rate in mm/min | FANUC actual feed register (actf) — mm/min |
| spindle_speed | Actual current spindle speed in RPM | FANUC actual spindle speed register (acts) |
| raw_cycle_time_sec | Elapsed time of the current part cycle in seconds (resets each cycle) | Timer in JS Script — resets on each part_count increment |
| part_count | Cumulative part counter from machine (never resets automatically) | FANUC macro variable #500 or part counter register (rdcnt) |
| last_cycle_time_sec | Actual time taken to complete the last full part cycle, in seconds | Captured in JS Script at each part_count increment event |
| idle_time_sec | Seconds the machine has been in RUNNING state but not cutting (feed = 0, spindle on) | JS Script timer — increments when is_running=true and feed_rate = 0 |
| connection_status | Communication link state between IIoT Gateway and controller: connected / disconnected | FOCAS2 API return code in JS Script |
| status | Overall data collection status: OK / ERROR | JS Script health check — OK if FOCAS2 returned valid data |

OEE Computed KPIs — Per Shift (ThingsBoard Push)

These parameters are calculated by the OEE Server (Node.js) by reading the oee_results table in the VM TimescaleDB. The SQL function populate_oee_today() runs every 5 minutes to recompute all shift KPIs from the raw cnc_data. The results are then pushed to ThingsBoard PE via REST API. Values are provided for both Shift A (07:00–19:00 IST) and Shift B (19:00–07:00 IST), denoted by the suffix _a or _b.

Push frequency: Every 5 minutes  |  Destination: ThingsBoard PE — REST telemetry endpoint

| Parameter (replace _a with _b for Shift B) | Meaning | Formula |
| --- | --- | --- |
| oee_shift_a | Overall Equipment Effectiveness for Shift A (%) | Availability × Performance × Quality ÷ 10,000 |
| availability_shift_a | Percentage of shift time the machine was running | Running seconds ÷ Net available seconds × 100 |
| performance_shift_a | How fast the machine ran vs its ideal (target) speed | (Parts produced × Target cycle sec) ÷ Running seconds × 100 |
| quality_shift_a | Percentage of produced parts that were accepted (no reject) | Good parts ÷ Total parts produced × 100 |
| parts_shift_a | Total parts produced during Shift A | MAX(part_count) − MIN(part_count) within shift window |
| target_shift_a | Target number of parts for Shift A (720 min shift) | Shift duration (720 min × 60 sec) ÷ Target cycle seconds |
| good_parts_shift_a | Total accepted (good) parts during Shift A | parts_shift_a − rejections_shift_a |
| rejections_shift_a | Number of rejected / scrapped parts during Shift A | COUNT of entries in cnc_rejection_log for that shift date and shift |
| cycle_eff_shift_a | How close the average cycle time was to the target cycle (%) | Target cycle seconds ÷ Avg cycle seconds × 100 |
| running_min_shift_a | Total minutes machine was in RUNNING / AUTO / BUSY state | COUNT(rows where machine_state = RUNNING) × 5 seconds ÷ 60 |
| downtime_min_shift_a | Total minutes machine was stopped, idle, or in alarm | 720 min − running_min_shift_a |
| alarms_shift_a | Number of distinct alarm events during Shift A | COUNT of consecutive alarm_active = true blocks in cnc_data |
| avg_cycle_shift_a | Average part cycle time in seconds for Shift A | AVG(last_cycle_time_sec) filtered to values between 5 and 3600 sec |
| target_cycle_shift_a | Ideal (target) cycle time per part in seconds | Loaded from program_master table for the active program_no |

OEE Daily Totals — Both Shifts Combined

These 8 parameters represent the combined performance across both Shift A and Shift B for the current calendar day (IST). They are calculated and pushed to ThingsBoard PE alongside the per-shift values, at the same 5-minute frequency.

Push frequency: Every 5 minutes  |  Destination: ThingsBoard PE — same REST telemetry payload

| Parameter | Meaning | Formula |
| --- | --- | --- |
| oee_today | Average OEE across both shifts today (%) | (oee_shift_a + oee_shift_b) ÷ number of completed shifts |
| availability_today | Average availability across both shifts today (%) | (availability_shift_a + availability_shift_b) ÷ 2 |
| performance_today | Average performance across both shifts today (%) | (performance_shift_a + performance_shift_b) ÷ 2 |
| quality_today | Average quality across both shifts today (%) | (quality_shift_a + quality_shift_b) ÷ 2 |
| parts_today | Total parts produced today — both shifts combined | parts_shift_a + parts_shift_b |
| target_today | Total target parts for today — both shifts combined | target_shift_a + target_shift_b |
| good_parts_today | Total accepted parts today — both shifts combined | good_parts_shift_a + good_parts_shift_b |
| rejections_today | Total rejected parts today — both shifts combined | rejections_shift_a + rejections_shift_b |